# Watts Up — Unified Requirements

Supersedes: watts-up-requirements.txt, watts-up-revision-1- through revision-7-requirements.txt

Last updated: 2026-05-12 (through Revision 7)

# 1. Program Overview

Single-file browser application for aircraft electrical load analysis. Organizes the aircraft power distribution system as a hierarchical tree.

Documents load changes associated with an aircraft modification and calculates net changes, new loads, and remaining capacity for each affected item.

# 2. Aircraft Metadata

Fields collected at startup and editable thereafter:

- Aircraft Make

- Aircraft Model

- Marketing Designation

- Serial Number

- Flight Phase / Scenario — free text, default: Cruise

# 3. Item Types

## 3.1 Root

Aircraft Total — available only when Parent = "No Parent (New Root)"

## 3.2 Generation

- Generator

- Alternator

- Battery

## 3.3 Conversion (Paired IN/OUT)

Each conversion device is added as two linked nodes: the **IN** node (input side) is the parent;

the **OUT** node (output side) is its child. Labels are suffixed "(IN)" and "(OUT)".

| Type | Input side | Output side | Default convPf |
| --- | --- | --- | --- |
| TRU | AC | DC | 0.95 |
| Inverter | DC | AC | 1.0 |
| Transformer | AC | AC | 1.0 |
| DC-DC Converter | DC | DC | 1.0 |
| Frequency Converter | AC | AC | 1.0 |

## 3.4 Distribution / Protection

- Feeder

- Bus

## 3.5 Distribution / Protection

- Circuit Breaker

- Fuse

## 3.6 Load

- Terminal node; has a Load Value field; no children allowed.

# 4. Power Units and Formulas

## 4.1 AC Items

Capacity and Remaining Capacity: Amps (A), Volt-Amps (VA)

All load columns (Existing Load, Load, Net Change, New Load): A, VA, W, VAR, pf

Formulas:

- VA = V × A = W / pf = √(W² + VAR²)

- W = VA × pf = √(VA² − VAR²)

- pf = W / VA; valid range 0 ≤ pf ≤ 1

- VAR = √(VA² − W²) = √(VA² − (VA × pf)²)

Default on save (if the user did not provide sufficient information): VAR = 0, pf = 1.

Applied only at save time; fields are left blank during entry.

AC vector-sum aggregation:

- Add W and VAR algebraically across children.

- Recalculate VA = √(W² + VAR²).

- Recalculate A = VA / V.

- Recalculate pf = W / VA.

## 4.2 DC Items

All columns (Capacity, Existing Load, Load, Net Change, New Load, Remaining): Amps (A), Watts (W)

Formulas: A = W / V; W = V × A

DC scalar-sum aggregation: Add amps; W = V × A.

# 5. Tree Display (Left-Hand Pane)

- Pre-order traversal; branches grouped under their parent node.

- "Root" label suppressed from display.

- Node label format: Description [RefDes] – RefDes in square brackets appended to the description, rendered in subdued secondary text; RefDes omitted if blank.

- Net change badge displayed per node:

  - AC items: VA value

  - DC items: A value

  - Positive NC: orange; Negative NC: green

- Collapse / expand toggle per node.

- Edit button on each node.

- Add Child button on each node — hidden for terminal nodes (Load type or Removed status).

# 6. Summary Table (Right-Hand Pane)

- Pre-order traversal matching the left-hand pane order.

- Description column uses the same Description [RefDes] format; there is no separate RefDes column.

- Column headers show units in parentheses, for example, "Capacity (VA)".

- Rows are color-coded for warnings (yellow) and overload (red).

## 6.1 DC Summary columns

| Capacity | Existing Load | Load | Net Change | New Load | Remaining |
| --- | --- | --- | --- | --- | --- |
| A, W | A, W | A, W | A, W | A, W | A, W |

## 6.2 AC Summary columns

| Capacity | Existing Load | Load | Net Change | New Load | Remaining |
| --- | --- | --- | --- | --- | --- |
| A, VA | A, VA, W, VAR, pf | A, VA, W, VAR, pf | A, VA, W, VAR, pf | A, VA, W, VAR, pf | A, VA |

## 6.3 Section ordering and labels

- Labels: DC Summary and AC Summary.

- If the first root node is AC, show AC Summary first, then DC Summary.

- If the first root node is DC, show DC Summary first, then AC Summary.

# 7. Edit Item Dialog

Previously named "Edit Item(s)"; renamed to "Edit Item" in Revision 7.

## 7.1 Field layout

- Parent Item dropdown (top of dialog)

  - Excludes children and descendants of the current item.

  - Includes "No Parent (New Root)" option.

- Item Type dropdown

  - "Aircraft Total" visible only when Parent = "No Parent (New Root)".

- Status: Existing / New / Removed

- Volts and AC / DC (see §7.2 for locking rules)

- Description (free text)

- RefDes (optional)

- Context-sensitive sections below (§7.3–7.9)

## 7.2 Volts and AC/DC locking rules

| Scenario | Volts | AC/DC |
| --- | --- | --- |
| Parent exists; regular (non-conv) node | Locked to parent value | Locked to parent value |
| Conv IN node; parent exists | Locked to parent value | Locked to device type |
| Conv IN node; no parent | Editable | Locked to device type |
| Conv OUT node | Editable | Locked to device type |
| No parent (new root); regular node | Editable | Editable |

## 7.3 Capacity section (non-load items)

- DC: A, W

- AC: A, VA

- Reset button: Deletes entries from the A and W fields (DC) or the A and VA fields (AC)

## 7.4 Existing Load section

- DC: A, W

- AC: Row 1 – A, VA; Row 2 – W, VAR, pf

- For new non-load items, this section is replaced by a Load field; the Load field is hidden if the item has children.

- Reset button: Deletes entries from the A and W fields (DC) or the A, VA, W, VAR, and pf fields (AC)

## 7.5 Load section (Load-type items only)

- DC: A, W

- AC: Row 1– A, VA; Row 2 – W, VAR, pf

- Reset button: Deletes entries from the A and W fields (DC) or the A, VA, W, VAR, and pf fields (AC)

## 7.6 Net Change Override section

- DC: W field + A field (A and W auto-derive from each other)

- AC: W field + VAR field (VAR defaults to 0 on save if left blank)

- Net change Override section is hidden if the item has children

- Reset button: Deletes entries from the A and W fields (DC) or the A, VA, W, VAR, and pf fields (AC)

## 7.7 Deleted

## 7.8 Deleted

## 7.9 Conversion parameters section (conversion items only)

- Efficiency input (default 0.85)

- convPf input — visible for TRU, Inverter, and Frequency Converter (default TRU = 0.95, Inverter and FC = 1.0)

- For Input Capacity and Existing Load, and Output Capacity and Existing Load:

  - DC: A, W (for DC inputs or outputs)

  - AC: A, VA (for DC inputs or outputs)

  - Reset button for Capacity, Existing Load, and Efficiency/Power Factor rows that delete all entries from the row

(TRU example:

Row 1 – Input Cap (A); Input Cap (VA); Output Cap (A); Output Cap (W); Reset button

Row 2 – Input Exist. Load (A); Input Exist. Load (VA); Output Exist. Load (A); Output Exist. Load (W); Reset button

Row 3 – Efficiency; Conv. Power Factor; Reset button)

- Recalculate fields on save

## 7.10 Duplicate button

- Copies the current node and appends "(copy)" to the description.

- For Conv IN nodes, also copies the paired OUT child node as a child of the copy.

## 7.11 Formula entry

All numeric fields accept Excel-style expressions starting with = (for example, =28*2 becomes 56).

# 8. Add Item(s) Dialog

## 8.1 Field layout

- Parent Item dropdown (top)

  - Excludes children and descendants of any selected item.

  - Includes "No Parent (New Root)" option.

- Item Type dropdown — "Aircraft Total" only when Parent = "No Parent"

- Status: Existing / New / Removed

- AC / DC selector — locked to the parent’s AC/DC when a parent is selected

- Master-detail table (§8.3)

## 8.2 State memory

- On open, clear existing table rows.

- Remember the last-used Item Type and Status, and restore them on next open.

## 8.3 Table columns (context-sensitive)

All row types include: Description, RefDes (optional).

| Item / Status | Additional columns |
| --- | --- |
| DC, any | Capacity (A or W), Existing Load (A, W), Load (A, W) |
| AC, any | Capacity (A, VA), Existing Load (A, VA, W, VAR, pf), Load (A, VA, W, VAR, pf) |
| Conv item | IN Volts column + OUT Volts column |
| Non-load, New | Load field replaces Existing Load; leave blank if the item will have children |

Formula entry applies to all numeric columns.

## 8.4 Volts and AC/DC locking

The same rules as Edit Item §7.2 apply, evaluated per row based on the selected parent and item type.

## 8.5 Root creation shortcut

+ Root button in the tree panel header opens the Add Item(s) dialog with Parent pre-set to "No Parent (New Root)".

# 9. Multiple Root Nodes

- Any number of independent root nodes allowed.

- Default root: type "Aircraft Total", Volts 115, AC.

- The last remaining root node cannot be deleted.

- Changing a node's parent to "No Parent (New Root)" promotes it to a root node.

- + Root button (§8.5) and Duplicate button (§7.10) are additional creation paths.

# 10. Calculations

## 10.1 Net Change (bottom-up propagation)

| Node condition | Net Change value |
| --- | --- |
| Load, status = New | loadValue |
| Load, status = Removed | −loadValue |
| Load, status = Existing | zero |
| Non-load, status = Removed | −existingLoad |
| Non-load with netChangeOverride | override value |
| Non-load, standard | sum of children NCs (passing through efficiency for conv children) |
| Conv IN node | derived from conv OUT NC via efficiency and convPf |

## 10.2 Derived totals

- New Load = existingLoad + netChange

- Remaining Capacity = capacity − newLoad

# 11. Warnings

| Condition | Warning |
| --- | --- |
| Negative load value entered | Warn on entry |
| New load is negative after net changes applied | Warn |
| New load exceeds item capacity | Warn |
| User adds children to a Load or Removed item | Warn |
| User adds children after manually entering a net change override | Warn |
| Non-load Removed item | Existing load entered as negative net change |

# 12. Data Persistence

- Export JSON: metadata + full node tree → file named WattsUp_{make}_{model}_{serial}_{date}.json

- Import JSON: restores a previously exported analysis.

- Auto-save to localStorage: state persisted across page refreshes.

# 13. Print

- Print-optimized layout triggered by the Print button.

- Header: aircraft metadata.

- AC Summary table and DC Summary table (ordered per §6.3).

- Warnings section.

# 14. Future Enhancements (Deferred)

- Three-phase AC circuit support.

- Multiple flight phases / scenarios (Takeoff, Cruise, Approach and Landing, Emergency, generator failure, etc.).

- Load intervals (instantaneous, 5-sec, 5-min, 15-min, continuous).

convPf applies to **all** AC conv-item calculations (capacity, existing load, load, net change,

new load). convPf field is visible in the dialog for TRU and Frequency Converter.

  - Distribution / Protection

- Feeder

- Bus

- Circuit Breaker

- Fuse

  - Load

Terminal node; has a Load Value field; no children allowed.

---

- ## 4. Power Units and Formulas

### 4.1 AC Items

**Capacity and Remaining Capacity:** Amps (A), Volt-Amps (VA)

**All load columns (Existing Load, Load, Net Change, New Load):** A, VA, W, VAR, pf

Formulas:

- VA = V × A = W / pf = √(W² + VAR²)

- W  = VA × pf = √(VA² − VAR²)

- pf = W / VA; valid range 0 ≤ pf ≤ 1

- VAR = √(VA² − W²) = √(VA² − (VA × pf)²)

Default on save (if user did not provide sufficient info): VAR = 0, pf = 1.

*(Applied only at save time; fields are left blank during entry.)*

**AC vector-sum aggregation:**

1. Add W and VAR algebraically across children

2. Recalculate VA = √(W² + VAR²)

3. Recalculate A = VA / V

4. Recalculate pf = W / VA

### 4.2 DC Items

**All columns (Capacity, Existing Load, Load, Net Change, New Load, Remaining):** Amps (A), Watts (W)

Formulas: A = W / V; W = V × A

**DC scalar-sum aggregation:** add amps; W = V × A.

---

- ## 5. Tree Display (Left-Hand Pane)

- Pre-order traversal; branches grouped under their parent node

- "Root" label suppressed from display

- Node label format: Description `[RefDes]` — RefDes in square brackets appended to description,

rendered in subdued (secondary) text; RefDes omitted if blank

- Net change badge displayed per node:

- AC items: VA value

- DC items: W value

- Positive NC: orange; Negative NC: green

- Collapse / expand toggle per node

- **Edit** button on each node

- **Add Child** button on each node — **hidden** for terminal nodes (Load type or Removed status)

---

- ## 6. Summary Table (Right-Hand Pane)

- Pre-order traversal matching the left-hand pane order

- Description column: same `Description [RefDes]` format; no separate RefDes column

- Column header units in parentheses, e.g., "Capacity (VA)"

- Rows color-coded for warnings (yellow) and overload (red)

### 6.1 DC Summary columns

| Capacity 	| Existing Load | Load 	| Net Change 	| New Load 	| Remaining 	|

|---		|---		|---	|---		|---		|---		|

| A, W 		| A, W 		| A, W 	| A, W 		| A, W 		| A, W 		|

### 6.2 AC Summary columns

| Capacity | Existing Load 	| Load 		    | Net Change 	| New Load 	    | Remaining |

|---	   |---			|---		    |---		|---		    |---        |

| A, VA    | A, VA, W, VAR, pf 	| A, VA, W, VAR, pf | A, VA, W, VAR, pf | A, VA, W, VAR, pf | A, VA     |

### 6.3 Section ordering and labels

- Labels: **"DC Summary"** and **"AC Summary"**

- If the first root node is AC: AC Summary first, then DC Summary

- If the first root node is DC: DC Summary first, then AC Summary

---

- ## 7. Edit Item Dialog

*(Previously named "Edit Item(s)"; renamed to "Edit Item" in Revision 7.)*

### 7.1 Field layout

1. **Parent Item** dropdown (top of dialog)

- Excludes children and descendants of the current item

- Includes "No Parent (New Root)" option

2. **Item Type** dropdown

- "Aircraft Total" visible only when Parent = "No Parent (New Root)"

3. **Status**: Existing / New / Removed

4. **Volts** and **AC / DC** (see §7.2 for locking rules)

5. Description (free text)

6. RefDes (optional)

7. Context-sensitive sections below (§7.3–7.9)

### 7.2 Volts and AC/DC locking rules

| Scenario | Volts | AC/DC |

|---|---|---|

| Parent exists; regular (non-conv) node | Locked to parent value | Locked to parent value |

| Conv IN node; parent exists | Locked to parent value | Locked to device type |

| Conv IN node; no parent | Editable | Locked to device type |

| Conv OUT node | Editable | Locked to device type |

| No parent (new root); regular node | Editable | Editable |

### 7.3 Capacity section (non-load items)

- DC: A, W

- AC: A, VA

### 7.4 Existing Load section

- DC: A, W

- AC: Row 1 — A, VA; Row 2 — W, VAR, pf

- For **new** non-load items: replaced by a **Load** field; Load field hidden if item has children

### 7.5 Load section (Load-type items only)

- DC: A, W

- AC: Row 1 — A, VA; Row 2 — W, VAR, pf

### 7.6 Net Change Override section

- DC: W field + A field (A and W auto-derive from each other)

- AC: W field + VAR field (VAR defaults to 0 on save if left blank)

### 7.7 AC field cascade derive (on field blur)

Applies to all AC field groups (Existing Load, Load). Requires VA to be set for W/VAR/pf

triggers; requires V > 0 for A/VA triggers. No-op if prerequisite values are missing.

| Field edited | Fields recalculated | Formulas |

|---|---|---|

| **A** | VA, W, VAR | VA = V×A; W = VA×pf; VAR = √(VA²−W²) |

| **VA** | A, W, VAR | A = VA/V; W = VA×pf; VAR = √(VA²−W²) |

| **W** | VAR, pf | VAR = √(VA²−W²); pf = W/VA |

| **VAR** | W, pf | W = √(VA²−VAR²); pf = W/VA |

| **pf** | W, VAR | W = VA×pf; VAR = √(VA²−W²) |

### 7.8 Cascade derive feedback

- Recalculated fields highlighted **yellow**

- Message displayed listing which fields were recalculated

- **OK** button for user acknowledgement

- After OK: yellow highlights removed from all fields; message text cleared; message hidden

### 7.9 Conversion parameters section (conversion items only)

- **Efficiency** input (default 0.85)

- **convPf** input — visible for TRU and Frequency Converter (default TRU = 0.95, FC = 1.0)

- IN/OUT capacity and load auto-derive via efficiency and convPf on input

### 7.10 Duplicate button

- Copies the current node; appends "(copy)" to the description

- For Conv IN nodes: also copies the paired OUT child node (child of the copy)

### 7.11 Formula entry

All numeric fields accept Excel-style expressions starting with `=` (e.g., `=28*2` → `56`).

---

- ## 8. Add Item(s) Dialog

### 8.1 Field layout

1. **Parent Item** dropdown (top)

- Excludes children/descendants of any selected item

- Includes "No Parent (New Root)" option

2. **Item Type** dropdown — "Aircraft Total" only when Parent = "No Parent"

3. **Status**: Existing / New / Removed

4. **AC / DC** selector — locked to parent's AC/DC when parent is selected

5. Master-detail table (§8.3)

### 8.2 State memory

- On open: clear existing table rows

- Remember last-used Item Type and Status; restore on next open

### 8.3 Table columns (context-sensitive)

All row types include: Description, RefDes (optional)

| Item / Status | Additional columns |

|---|---|

| DC, any | Capacity (A or W), Existing Load (A, W), Load (A, W) |

| AC, any | Capacity (A, VA), Existing Load (A, VA, W, VAR, pf), Load (A, VA, W, VAR, pf) |

| Conv item | IN Volts column + OUT Volts column |

| Non-load, New | Load field replaces Existing Load; leave blank if item will have children |

Formula entry applies to all numeric columns.

### 8.4 Volts and AC/DC locking

Same rules as Edit Item §7.2; applied per row based on selected parent and item type.

### 8.5 Root creation shortcut

**"+ Root" button** in the tree panel header opens the Add Item(s) dialog with

Parent pre-set to "No Parent (New Root)".

---

- Multiple Root Nodes

- Any number of independent root nodes allowed

- Default root: type "Aircraft Total", Volts 115, AC

- Last remaining root node cannot be deleted

- Changing a node's parent to "No Parent (New Root)" promotes it to a root node

- "+ Root" button (§8.5) and Duplicate button (§7.10) are additional creation paths

---

- Calculations

### 10.1 Net Change (bottom-up propagation)

| Node condition | Net Change value |

|---|---|

| Load, status = New | loadValue |

| Load, status = Removed | −loadValue |

| Load, status = Existing | zero |

| Non-load, status = Removed | −existingLoad |

| Non-load with netChangeOverride | override value |

| Non-load, standard | sum of children NCs (passing through efficiency for conv children) |

| Conv IN node | derived from conv OUT NC via efficiency and convPf |

### 10.2 Derived totals

- **New Load** = existingLoad + netChange

- **Remaining Capacity** = capacity − newLoad

---

- Warnings

| Condition | Warning |

|---|---|

| Negative load value entered | Warn on entry |

| New load is negative after net changes applied | Warn |

| New load exceeds item capacity | Warn |

| User adds children to a Load or Removed item | Warn |

| User adds children after manually entering a net change override | Warn |

| Non-load Removed item | Existing load entered as negative net change |

---

- Data Persistence

- **Export JSON**: metadata + full node tree → file named `WattsUp_{make}_{model}_{serial}_{date}.json`

- **Import JSON**: restores a previously exported analysis

- **Auto-save to localStorage**: state persisted across page refreshes

---

- Print

- Print-optimized layout triggered by Print button

- Header: aircraft metadata

- AC Summary table and DC Summary table (ordered per §6.3)

- Warnings section

---

- Future Enhancements (Deferred)

- Three-phase AC circuit support

- Multiple flight phases / scenarios (Takeoff, Cruise, Approach and Landing, Emergency,

generator failure, etc.)

- Load intervals (instantaneous, 5-sec, 5-min, 15-min, continuous)