## Revision 7

==========================================

Note: Backward compatibility with data from the previous version is desired but not required.

Start with updates and corrections to the Edit Item(s) dialog box.

  - Change the name of the Edit Item(s) dialog box to “Edit Item”.

  - Retain the following (previously implemented) automatic calculations when the user over-rides data:

    - IF user manually over-rides Amps value THEN

- re-calculate VA (VA=Volts * Amps);

- re-calculate Watts (W = VA * pf)

- re-calculate VAR (VAR = sqrt(VA^2 - W^2))

    - IF user manually over-rides VA value THEN

- re-calculate Amps (A=VA / V);

- re-calculate Watts (Watts = VA * pf)

- re-calculate VAR (VAR = sqrt(VA^2 - W^2))

    - IF user manually over-rides Watts value THEN

- re-calculate VAR (VAR = sqrt(VA^2 - W^2));

- re-calculate pf (pf = W / VA)

  - In addition to the calculations listed under item 2 above, incorporate the following:

    - IF user manually over-rides VAR value THEN

- re-calculate Watts (Watts = sqrt(VA^2 - VAR^2));

- re-calculate pf (pf = (sqrt(VA^2 - VAR^2))/ VA)

    - IF user manually over-rides pf value THEN

- re-calculate Watts (Watts = VA * pf);

- re-calculate VAR (VAR = sqrt(VA^2 – (VA*pf)^2))

  - Retain the following (previously implemented) when the user over-rides data:

    - highlight re-calculated fields

    - display a message indicating which fields were re-calculated

    - Request user acknowledgement

  - Modify item 4 above with the following:

    - After user acknowledgement, remove the highlighting from the recalculated fields and clear the message indicating which field was re-calculated